R8 Assembly Language

Programming Guide & Complete ISA Reference

R816 / R824 / R832 — BASE ISA v9 — Platform v15

108 BASE opcodes + EXT0 R8BLK block operations

March 2026

# Part I — Programmer's Guide

## Chapter 1: Introduction to R8

R8 is a word-sized stack machine ISA designed for FPGA soft-cores, teaching, and microarchitecture research. It scales from 16-bit (R816) through 24-bit (R824) to 32-bit (R832) by changing a single parameter: WORD_BITS. All operand-stack values are native WORD size.

The architecture combines ideas from three lineages: the Transputer's three-register stack cache (A/B/C), RISC-V's clean fielded encoding and privilege model, and the 'Nintendo rule' — add only primitives that provide at least 5x improvement over the software alternative.

The result is an ISA small enough to hold in your head (108 opcodes), structured enough for a real compiler backend, and simple enough to implement in Verilog over a weekend.

### 1.1 Design Principles

No flags: R8 has no condition flags register. Comparisons produce 0/1 values on the stack (SEQ, SLT, etc.). Multi-word carry is an explicit stack value (ADDC/SUBB). This makes interrupts trivially safe and simplifies out-of-order execution research.

Micro-fusion, not macro-fusion: rather than dedicating opcodes to fused load-operate patterns, R8 defines clean single-operation instructions and lets the implementation fuse common pairs (B imm8; ADD becomes add-immediate internally).

BASE is frozen: all future instructions go through the ESC extension mechanism. The 108 BASE opcodes will never change, ensuring backward compatibility.

### 1.2 Word Widths

| Variant | WORD_BITS | WORD_BYTES | Address Space | I instruction length |
| --- | --- | --- | --- | --- |
| R816 | 16 | 2 | 64 KB | 3 bytes (1+2) |
| R824 | 24 | 3 | 16 MB | 4 bytes (1+3) |
| R832 | 32 | 4 | 4 GB | 5 bytes (1+4) |

## Chapter 2: The Programming Model

### 2.1 The Operand Stack

R8's primary computation model is a stack. Values are pushed onto the stack, operations consume values from the top and push results. The top three stack positions are cached in hardware registers named A (TOS), B, and C.

Most instructions operate on A and/or B and leave the result in A. For example, ADD pops both A and B, computes B+A, and pushes the result into A (with C promoted to B).

The stack is not unlimited. Programs should not assume more than 3 values can be held without spilling to workspace or memory. The compiler and programmer manage stack depth explicitly.

### 2.2 Workspace Locals

The workspace is a block of 256 local variable slots (wksp[0..255]) that serve as the function's register file. Slots 0-15 have dedicated 1-byte opcodes (LDL0-LDL15, STL0-STL15). Slots 16-255 use the 2-byte LDLX/STLX instructions.

Workspace access is the primary mechanism for managing values beyond the 3-deep stack cache. Instead of stack shuffling (like Forth's ROT/OVER), R8 programmers store intermediate values in workspace slots and reload them when needed.

### 2.3 ABI Workspace Assignments

| Register | Role | Convention |
| --- | --- | --- |
| w0-w3 | Arguments / return value | Caller-saved. w0 is also the return value. |
| w4-w13 | Scratch / locals | Caller-saved. Functions may use freely. |
| w14 | LR (Link Register) | Callee-saved via PUSH/POP. |
| w15 (SP) | Stack Pointer | Hardware-managed by PUSH/POP. |

### 2.4 Memory

R8 uses a flat, byte-addressed memory space. Memory is little-endian for all multi-byte accesses. The address space size depends on the word width: 64KB for R816, 16MB for R824, 4GB for R832.

The memory stack (accessed via PUSH/POP) grows downward: PUSH decrements SP then writes, POP reads then increments SP. SP is wksp[15].

## Chapter 3: Assembly Language Syntax

### 3.1 Basic Format

Each line contains an optional label, an instruction mnemonic, and operands:

label:    mnemonic operand     ; comment

start:    u 0x02              ; push syscall number

stl w0              ; store to workspace slot 0

ecall               ; system call

hlt                 ; halt

### 3.2 Operand Formats

Workspace registers: w0 through w14 and sp (alias for w15). Used with LDL/STL instructions.

Immediate values: decimal (42), hexadecimal (0x2A or $2A), with optional underscores (0xFF_00).

Labels: symbolic names followed by a colon. Used as branch/jump targets. The assembler resolves them to addresses or offsets.

Expressions: full C-like expression evaluation: +, -, *, /, %, <<, >>, &, |, ^, ~, !, &&, ||, comparisons, parentheses. The special symbol '.' refers to the current PC.

### 3.3 Directives

.org addr        ; set origin address

.db val, ...     ; emit byte(s)

.dw val, ...     ; emit word(s)

.ds count        ; reserve count bytes of space

.equ name, expr  ; define constant symbol

.include "file"  ; include source file

## Chapter 4: Common Patterns

### 4.1 Function Call and Return

; Calling a function:

jal my_function     ; push return addr, jump

; Function prologue:

my_function:

stl w14             ; save return address to LR

; ... function body ...

ldl w14             ; load return address

jr                  ; return

### 4.2 Nested Calls (saving LR)

outer:

stl w14             ; save outer return address

ldl w14; push       ; save LR to memory stack

jal inner           ; call (overwrites LR)

pop; stl w14        ; restore outer LR

ldl w14; jr         ; return to outer caller

### 4.3 If/Else

; if (x == 0) { ... } else { ... }

ldl w4              ; load x

bnez .else          ; branch if x != 0

; ... then block ...

bra .endif

.else:

; ... else block ...

.endif:

### 4.4 Counted Loop

; for (i = 10; i > 0; i--)

u 10

stl w4              ; w4 = i = 10

.loop:

; ... loop body using w4 ...

ldl w4; dec; stl w4 ; i--

ldl w4; bnez .loop  ; if i != 0, repeat

### 4.5 Struct Field Access

; Load field at offset 6 from base pointer in w5

ldl w5              ; push base address

u 6                 ; push offset

add                 ; compute address

ld                  ; load the field value

; (4 bytes, 4 instructions — micro-fuses to ~1 uop on console cores)

### 4.6 32-bit Add on R816

; r = a + b  where a=(w0:w1), b=(w2:w3), r=(w4:w5)

; w0=a_lo, w1=a_hi, w2=b_lo, w3=b_hi

ldl w0; ldl w2

addc               ; -> carry, lo_sum

stl w4             ; w4 = lo_sum, carry on stack

ldl w1; add        ; carry + a_hi

ldl w3; add        ; + b_hi

stl w5             ; w5 = hi result

### 4.7 16x16 -> 32 Multiply on R816

ldl w0; ldl w1; mul;   stl w4   ; low 16 bits

ldl w0; ldl w1; mulhu; stl w5   ; high 16 bits

### 4.8 Block Copy (R8BLK)

; memcpy(dst, src, nbytes) using MOVB

; push dst, src, n onto stack, then store to w0-w2

stl w2             ; n -> w2

stl w1             ; src -> w1

stl w0             ; dst -> w0

movb               ; ESC 0x01 — restartable block copy

### 4.9 System Call

; SYS_PUTC (print character 'A')

u 0x04             ; w0 = SYS_PUTC

stl w0

u 65               ; w1 = 'A'

stl w1

ecall              ; trap to BIOS handler

# Part II — Complete Instruction Set Reference

This section documents every R8 BASE instruction with its encoding, stack effect, and detailed behavior. Instructions are grouped by category.

## Stack Manipulation

### NOP  (0x00)

| Property | Value |
| --- | --- |
| Opcode | 0x00 |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=000 fmt=00 |
| Category | OP / STACK |

Description: No operation. Does nothing; advances PC by 1 byte.

Notes: Used for alignment padding or delay slots.

### DUP  (0x04)

| Property | Value |
| --- | --- |
| Opcode | 0x04 |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, x, x |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=001 fmt=00 |
| Category | OP / STACK |

Description: Duplicate the top-of-stack value. A copy of A is pushed; B becomes the old A, C becomes the old B.

Notes: Essential for using a value twice without reloading from workspace.

### SWAP  (0x08)

| Property | Value |
| --- | --- |
| Opcode | 0x08 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, y, x |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=010 fmt=00 |
| Category | OP / STACK |

Description: Exchange the top two stack values. A and B are swapped; C is unchanged.

Notes: Used to reorder operands for non-commutative operations (SUB, DIV, SLT).

### DROP1  (0x0C)

| Property | Value |
| --- | --- |
| Opcode | 0x0C |
| Length | 1 byte(s) |
| Stack Effect | …, x → … |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=011 fmt=00 |
| Category | OP / STACK |

Description: Discard the top-of-stack value. A is replaced by B, B by C.

Notes: Use after operations that leave an unwanted result on the stack.

### DROP2  (0x10)

| Property | Value |
| --- | --- |
| Opcode | 0x10 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=100 fmt=00 |
| Category | OP / STACK |

Description: Discard the top two stack values. A is replaced by C; B and C become zero.

Notes: Efficient cleanup after two-operand operations. B and C are architecturally zero after this instruction.

### I2B  (0x14)

| Property | Value |
| --- | --- |
| Opcode | 0x14 |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, sext8(x) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=101 fmt=00 |
| Category | OP / STACK |

Description: Sign-extend the low 8 bits of A to the full native word width. Bits 8 and above are replaced by copies of bit 7.

Notes: Use after byte arithmetic to restore correct signed interpretation.

## Arithmetic

### ADD  (0x20)

| Property | Value |
| --- | --- |
| Opcode | 0x20 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x+y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=000 fmt=00 |
| Category | OP / ARITH |

Description: Add the top two values. Result = B + A (modular, wraps on overflow). Both operands consumed; C promoted.

Notes: The most common instruction in compiled code. Micro-fuses with preceding B/U/I immediate.

### SUB  (0x24)

| Property | Value |
| --- | --- |
| Opcode | 0x24 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x-y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=001 fmt=00 |
| Category | OP / ARITH |

Description: Subtract A from B. Result = B - A (modular). Note: the value pushed first (deeper) is the minuend.

Notes: Operand order: push x, push y, SUB gives x-y. Use SWAP before SUB if you need y-x.

### MUL  (0x28)

| Property | Value |
| --- | --- |
| Opcode | 0x28 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x*y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=010 fmt=00 |
| Category | OP / ARITH |

Description: Multiply, keeping the low WORD_BITS of the product. Result = low(B * A). Both signed and unsigned produce the same low bits.

Notes: For the upper half of the product, use MULH (signed) or MULHU (unsigned).

### DIV  (0x2C)

| Property | Value |
| --- | --- |
| Opcode | 0x2C |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x/y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=011 fmt=00 |
| Category | OP / ARITH |

Description: Signed integer division. Result = B / A, truncated toward zero (C99/Java semantics). Traps TRAP_DIV_ZERO if A == 0.

Notes: Division truncation: 7/2=3, -7/2=-3. Remainder sign follows dividend (see REM).

### REM  (0x30)

| Property | Value |
| --- | --- |
| Opcode | 0x30 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x%y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=100 fmt=00 |
| Category | OP / ARITH |

Description: Signed integer remainder. Result = B % A. Sign of result matches sign of B (dividend). Traps if A == 0.

Notes: Identity: (x/y)*y + (x%y) == x always holds.

### INC  (0x34)

| Property | Value |
| --- | --- |
| Opcode | 0x34 |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, x+1 |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=101 fmt=00 |
| Category | OP / ARITH |

Description: Increment A by 1. Equivalent to B 1; ADD but in 1 byte instead of 3.

Notes: Common in loop counters and pointer advancement.

### DEC  (0x38)

| Property | Value |
| --- | --- |
| Opcode | 0x38 |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, x-1 |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=110 fmt=00 |
| Category | OP / ARITH |

Description: Decrement A by 1.

Notes: Common in countdown loops.

### NEG  (0x3C)

| Property | Value |
| --- | --- |
| Opcode | 0x3C |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, -x |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=111 fmt=00 |
| Category | OP / ARITH |

Description: Two's complement negate. Result = 0 - A. NEG of the minimum signed value (e.g., -32768 on R816) returns itself.

Notes: Useful for computing absolute values: DUP; B 0; SLT; BNEZ .pos; NEG; .pos:

### ADDC  (0x80)

| Property | Value |
| --- | --- |
| Opcode | 0x80 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, carry, sum |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=1 f2=00 f3=000 fmt=00 |
| Category | OP / ARITH |

Description: Add with carry-on-stack. Computes x+y; pushes both the carry (0 or 1) and the sum. Carry is below sum so STL can store the sum and leave carry accessible for the next limb.

Notes: Nintendo primitive. No flags register needed. For 32-bit add on R816: ADDC for low limb, then ADD carry into high limb. Interrupts between limbs are safe because carry is an explicit stack value.

### SUBB  (0x84)

| Property | Value |
| --- | --- |
| Opcode | 0x84 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, borrow, diff |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=1 f2=00 f3=001 fmt=00 |
| Category | OP / ARITH |

Description: Subtract with borrow-on-stack. Computes x-y; pushes borrow (1 if x < y unsigned, else 0) and difference. Borrow is below diff.

Notes: Nintendo primitive. Borrow chaining for multi-limb subtraction. Same interrupt-safe property as ADDC.

### MULH  (0x88)

| Property | Value |
| --- | --- |
| Opcode | 0x88 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, hi |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=1 f2=00 f3=010 fmt=00 |
| Category | OP / ARITH |

Description: Signed upper-half multiply. Returns bits [2W-1:W] of the signed product x*y. Use with MUL (low half) to form full 2*WORD products.

Notes: Nintendo primitive. For 16x16->32 on R816: MUL gives low 16, MULH gives high 16 (signed).

### MULHU  (0x8C)

| Property | Value |
| --- | --- |
| Opcode | 0x8C |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, hi |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=1 f2=00 f3=011 fmt=00 |
| Category | OP / ARITH |

Description: Unsigned upper-half multiply. Returns bits [2W-1:W] of the unsigned product u(x)*u(y).

Notes: Nintendo primitive. For 16x16->32 unsigned on R816: MUL gives low 16, MULHU gives high 16.

### DIVU  (0xAC)

| Property | Value |
| --- | --- |
| Opcode | 0xAC |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, u(x)/u(y) |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=1 f2=01 f3=011 fmt=00 |
| Category | OP / ARITH |

Description: Unsigned integer division. Both operands treated as unsigned. Traps if A == 0.

Notes: Essential for address arithmetic and array index calculations.

### REMU  (0xB0)

| Property | Value |
| --- | --- |
| Opcode | 0xB0 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, u(x)%u(y) |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=1 f2=01 f3=100 fmt=00 |
| Category | OP / ARITH |

Description: Unsigned integer remainder. Both operands treated as unsigned. Traps if A == 0.

Notes: Used in hash tables, modular arithmetic, and printf decimal conversion.

## Bit Manipulation

### CLZ  (0x90)

| Property | Value |
| --- | --- |
| Opcode | 0x90 |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, n |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=1 f2=00 f3=100 fmt=00 |
| Category | OP / BIT |

Description: Count leading zeros. Returns the number of consecutive zero bits starting from the MSB. CLZ(0) = WORD_BITS (normative).

Notes: Nintendo primitive. Critical for soft-float normalization (avoids shift loops), integer log2, priority encoding, and allocator bitmap scanning. Hardware cost: one priority-encoder tree (~8 LUTs on R816).

## Comparison

### SEQ  (0x50)

| Property | Value |
| --- | --- |
| Opcode | 0x50 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, (x==y)?1:0 |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=100 fmt=00 |
| Category | OP / CMP |

Description: Set if equal. Pushes 1 if B equals A, else 0. Both operands consumed.

Notes: Produces a boolean (0/1) on the stack for use with BNEZ/BEQZ or as a computed value.

### SNE  (0x54)

| Property | Value |
| --- | --- |
| Opcode | 0x54 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, (x!=y)?1:0 |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=101 fmt=00 |
| Category | OP / CMP |

Description: Set if not equal. Pushes 1 if B does not equal A, else 0.

### SLT  (0x6C)

| Property | Value |
| --- | --- |
| Opcode | 0x6C |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, (x<y)?1:0 |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=11 f3=011 fmt=00 |
| Category | OP / CMP |

Description: Set if less than (signed comparison). Pushes 1 if signed(B) < signed(A), else 0.

Notes: RISC-V aligned. For >=: SLT then B 1; XOR. For >: SWAP; SLT.

### SLTU  (0x70)

| Property | Value |
| --- | --- |
| Opcode | 0x70 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, (u(x)<u(y))?1:0 |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=11 f3=100 fmt=00 |
| Category | OP / CMP |

Description: Set if less than (unsigned comparison). Pushes 1 if unsigned(B) < unsigned(A), else 0.

Notes: Used for address comparisons, bounds checking, and carry detection in software multi-word arithmetic.

## Bitwise Logic

### AND  (0x40)

| Property | Value |
| --- | --- |
| Opcode | 0x40 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x&y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=000 fmt=00 |
| Category | OP / LOGIC |

Description: Bitwise AND of B and A.

Notes: Common for masking bits. E.g., U 0x0F; AND extracts the low nibble.

### OR  (0x44)

| Property | Value |
| --- | --- |
| Opcode | 0x44 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x¦y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=001 fmt=00 |
| Category | OP / LOGIC |

Description: Bitwise OR of B and A.

Notes: Common for setting flag bits.

### XOR  (0x48)

| Property | Value |
| --- | --- |
| Opcode | 0x48 |
| Length | 1 byte(s) |
| Stack Effect | …, x, y → …, x^y |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=010 fmt=00 |
| Category | OP / LOGIC |

Description: Bitwise XOR of B and A.

Notes: XOR with all-1s is equivalent to INV. B 1; XOR is the canonical boolean NOT.

### INV  (0x4C)

| Property | Value |
| --- | --- |
| Opcode | 0x4C |
| Length | 1 byte(s) |
| Stack Effect | …, x → …, ~x |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=011 fmt=00 |
| Category | OP / LOGIC |

Description: Bitwise complement (one's complement). Flips all bits of A.

Notes: Note: INV is bitwise NOT, not boolean NOT. For boolean: B 1; XOR.

## Shift

### SHL  (0x60)

| Property | Value |
| --- | --- |
| Opcode | 0x60 |
| Length | 1 byte(s) |
| Stack Effect | …, x, s → …, x<<s |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=11 f3=000 fmt=00 |
| Category | OP / SHIFT |

Description: Shift B left by (A & (WORD_BITS-1)) positions. Vacated low bits are filled with zeros.

Notes: The mask prevents undefined behavior for shift counts >= WORD_BITS. Micro-fuses with preceding immediate for constant shifts.

### SHR  (0x64)

| Property | Value |
| --- | --- |
| Opcode | 0x64 |
| Length | 1 byte(s) |
| Stack Effect | …, x, s → …, u(x)>>>s |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=11 f3=001 fmt=00 |
| Category | OP / SHIFT |

Description: Logical (unsigned) right shift. Shift B right by masked amount. Vacated high bits filled with zeros.

Notes: Use for unsigned division by powers of 2 and bit extraction.

### SAR  (0x68)

| Property | Value |
| --- | --- |
| Opcode | 0x68 |
| Length | 1 byte(s) |
| Stack Effect | …, x, s → …, x>>s |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=11 f3=010 fmt=00 |
| Category | OP / SHIFT |

Description: Arithmetic (signed) right shift. Vacated high bits filled with copies of the sign bit (MSB).

Notes: Use for signed division by powers of 2 (rounds toward negative infinity, not toward zero like DIV).

## 8-bit Immediates

### B  (0x02)

| Property | Value |
| --- | --- |
| Opcode | 0x02 |
| Length | 2 byte(s) |
| Stack Effect | … → …, sext8(imm8) |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=000 fmt=10 |
| Category | IMM / IMM8 |

Description: Push signed 8-bit immediate (-128 to +127). The byte following the opcode is sign-extended to WORD width and pushed.

Notes: Most common immediate form. 'B' stands for 'Byte literal' (not Branch). Micro-fuses with following ALU op to form add-immediate etc.

### U  (0x06)

| Property | Value |
| --- | --- |
| Opcode | 0x06 |
| Length | 2 byte(s) |
| Stack Effect | … → …, zext8(imm8) |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=001 fmt=10 |
| Category | IMM / IMM8 |

Description: Push unsigned 8-bit immediate (0 to 255). The byte following the opcode is zero-extended to WORD width and pushed.

Notes: Use when the value is positive and may exceed 127 (where B would sign-extend incorrectly).

## Word Immediates

### I  (0x43)

| Property | Value |
| --- | --- |
| Opcode | 0x43 |
| Length | 1+W byte(s) |
| Stack Effect | … → …, immW |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=000 fmt=11 |
| Category | IMM / IMMW |

Description: Push native-word-sized immediate. The opcode is followed by WORD_BYTES of little-endian immediate data.

Notes: Total length: 3 bytes on R816, 4 on R824, 5 on R832. Use for constants that don't fit in 8 bits.

## PC-Relative Address

### AIIP  (0x47)

| Property | Value |
| --- | --- |
| Opcode | 0x47 |
| Length | 1+W byte(s) |
| Stack Effect | … → …, nextPC+immW |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=001 fmt=11 |
| Category | IMM / PCREL |

Description: PC-relative address load. Pushes the address computed as (PC after this instruction) + signed immW. Used to load addresses of data or code relative to the current position.

Notes: 'AIIP' = Address Immediate Instruction Pointer. Equivalent to RISC-V AUIPC but loads the complete address in one instruction.

## Word Load

### LD  (0x01)

| Property | Value |
| --- | --- |
| Opcode | 0x01 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, val |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=000 fmt=01 |
| Category | LOAD / WORD |

Description: Load a native WORD from memory. Replaces A (the address) with the WORD value at that address. Little-endian byte order.

Notes: The fundamental memory read. On R816, reads 2 bytes; on R832, reads 4 bytes.

## Narrow Loads

### LB  (0x05)

| Property | Value |
| --- | --- |
| Opcode | 0x05 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, sext8(mem8[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=001 fmt=01 |
| Category | LOAD / NARROW |

Description: Load a byte and sign-extend to WORD width.

Notes: Use for signed char/byte data.

### LU  (0x09)

| Property | Value |
| --- | --- |
| Opcode | 0x09 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, zext8(mem8[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=010 fmt=01 |
| Category | LOAD / NARROW |

Description: Load a byte and zero-extend to WORD width.

Notes: Use for unsigned char/byte data (pixel values, ASCII characters).

### LH  (0x0D)

| Property | Value |
| --- | --- |
| Opcode | 0x0D |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, sextH(memH[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=011 fmt=01 |
| Category | LOAD / NARROW |

Description: Load a 16-bit halfword and sign-extend. Width guard: valid only when WORD_BYTES >= 4. Traps ILLEGAL_INSN on R816.

Notes: For R832 accessing 16-bit data (VPU registers, audio samples).

### LHU  (0x11)

| Property | Value |
| --- | --- |
| Opcode | 0x11 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, zextH(memH[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=100 fmt=01 |
| Category | LOAD / NARROW |

Description: Load a 16-bit halfword and zero-extend. Same width guard as LH.

### L32  (0x15)

| Property | Value |
| --- | --- |
| Opcode | 0x15 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, sext32(mem32[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=101 fmt=01 |
| Category | LOAD / NARROW |

Description: Load a 32-bit value and sign-extend. Width guard: valid only when WORD_BYTES > 4 (R864). Traps on R816/R824/R832.

Notes: Reserved for future R864 implementations.

### L32U  (0x19)

| Property | Value |
| --- | --- |
| Opcode | 0x19 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, zext32(mem32[addr]) |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=00 f3=110 fmt=01 |
| Category | LOAD / NARROW |

Description: Load 32-bit and zero-extend. Same guard as L32.

## Word Store

### ST  (0x21)

| Property | Value |
| --- | --- |
| Opcode | 0x21 |
| Length | 1 byte(s) |
| Stack Effect | …, addr, val → … |
| Stack Delta | -2 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=000 fmt=01 |
| Category | STORE / WORD |

Description: Store a native WORD to memory. B is the address, A is the value. Both consumed (STACKD=-2). Little-endian.

Notes: Fire-and-forget: no stale address left on stack (unlike Transputer's ST which left the address).

## Narrow Stores

### SB  (0x25)

| Property | Value |
| --- | --- |
| Opcode | 0x25 |
| Length | 1 byte(s) |
| Stack Effect | …, addr, val → … |
| Stack Delta | -2 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=001 fmt=01 |
| Category | STORE / NARROW |

Description: Store the low byte of A to the address in B. Both consumed.

### SH  (0x29)

| Property | Value |
| --- | --- |
| Opcode | 0x29 |
| Length | 1 byte(s) |
| Stack Effect | …, addr, val → … |
| Stack Delta | -2 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=010 fmt=01 |
| Category | STORE / NARROW |

Description: Store a 16-bit halfword. Width guard: WORD_BYTES >= 4.

### S32  (0x2D)

| Property | Value |
| --- | --- |
| Opcode | 0x2D |
| Length | 1 byte(s) |
| Stack Effect | …, addr, val → … |
| Stack Delta | -2 |
| Barrier | No |
| Encoding | b7=0 f2=01 f3=011 fmt=01 |
| Category | STORE / NARROW |

Description: Store 32-bit. Width guard: WORD_BYTES > 4.

## Memory Stack (PUSH/POP)

### POP  (0x41)

| Property | Value |
| --- | --- |
| Opcode | 0x41 |
| Length | 1 byte(s) |
| Stack Effect | … → …, val |
| Stack Delta | +1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=000 fmt=01 |
| Category | MEM / STACKMEM |

Description: Pop from memory stack. Reads a WORD from mem[SP], increments SP by WORD_BYTES, and pushes the value.

Notes: SP is wksp[15]. Memory stack grows downward (PUSH decrements, POP increments).

### PUSH  (0x45)

| Property | Value |
| --- | --- |
| Opcode | 0x45 |
| Length | 1 byte(s) |
| Stack Effect | …, x → … |
| Stack Delta | -1 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=001 fmt=01 |
| Category | MEM / STACKMEM |

Description: Push to memory stack. Decrements SP by WORD_BYTES, writes A to mem[SP], then drops A from operand stack.

Notes: Used to save workspace registers across calls (LDL14; PUSH to save LR).

## Conditional Branches

### BEQ  (0x22)

| Property | Value |
| --- | --- |
| Opcode | 0x22 |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=000 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if equal. If B == A, jump to PC + sext8(imm8). Both operands consumed. B and C become zero.

Notes: Offset is relative to the byte AFTER the 2-byte instruction. Range: -128 to +127 bytes from nextPC.

### BNE  (0x26)

| Property | Value |
| --- | --- |
| Opcode | 0x26 |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=001 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if not equal. Jump if B != A.

### BLT  (0x2A)

| Property | Value |
| --- | --- |
| Opcode | 0x2A |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=010 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if less than (signed). Jump if signed(B) < signed(A).

### BLTU  (0x2E)

| Property | Value |
| --- | --- |
| Opcode | 0x2E |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=011 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if less than (unsigned). Jump if unsigned(B) < unsigned(A).

### BGE  (0x32)

| Property | Value |
| --- | --- |
| Opcode | 0x32 |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=100 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if greater or equal (signed). Jump if signed(B) >= signed(A).

### BGEU  (0x36)

| Property | Value |
| --- | --- |
| Opcode | 0x36 |
| Length | 2 byte(s) |
| Stack Effect | …, x, y → … |
| Stack Delta | -2 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=101 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if greater or equal (unsigned). Jump if unsigned(B) >= unsigned(A).

### BEQZ  (0x3A)

| Property | Value |
| --- | --- |
| Opcode | 0x3A |
| Length | 2 byte(s) |
| Stack Effect | …, x → … |
| Stack Delta | -1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=110 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if zero. Jump if A == 0. Only one operand consumed (STACKD=-1).

Notes: The most common branch: testing a boolean/flag/null. 1 byte shorter than B 0; BEQ.

### BNEZ  (0x3E)

| Property | Value |
| --- | --- |
| Opcode | 0x3E |
| Length | 2 byte(s) |
| Stack Effect | …, x → … |
| Stack Delta | -1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=01 f3=111 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Branch if not zero. Jump if A != 0.

Notes: Second most common branch.

### BRA  (0xA2)

| Property | Value |
| --- | --- |
| Opcode | 0xA2 |
| Length | 2 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=1 f2=01 f3=000 fmt=10 |
| Category | BR/JMP / BRANCH |

Description: Unconditional branch. Always jumps to PC + sext8(imm8). No stack operands consumed.

Notes: Used for short unconditional jumps (if/else skip, loop back-edge). For longer jumps, use J.

## Jumps and Calls

### JR  (0x42)

| Property | Value |
| --- | --- |
| Opcode | 0x42 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → … |
| Stack Delta | -1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=000 fmt=10 |
| Category | BR/JMP / JUMP |

Description: Jump register. Sets PC = A. The address is consumed from the stack.

Notes: Used for function returns: LDL14; JR (load LR, jump to it).

### JALR  (0x46)

| Property | Value |
| --- | --- |
| Opcode | 0x46 |
| Length | 1 byte(s) |
| Stack Effect | …, addr → …, ret |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=001 fmt=10 |
| Category | BR/JMP / JUMP |

Description: Call register (indirect). Saves nextPC as return address, jumps to A. Replaces A with the return address (STACKD=0).

Notes: Used for function pointers and virtual dispatch. Callee stores return addr: STL14.

### J  (0x73)

| Property | Value |
| --- | --- |
| Opcode | 0x73 |
| Length | 1+W byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=100 fmt=11 |
| Category | BR/JMP / JUMP |

Description: Relative jump (long). PC = nextPC + signed immW. No stack effect.

Notes: Used for long unconditional jumps that exceed BRA's +/-127 byte range. Also used in long-branch trampolines.

### JAL  (0x77)

| Property | Value |
| --- | --- |
| Opcode | 0x77 |
| Length | 1+W byte(s) |
| Stack Effect | … → …, ret |
| Stack Delta | +1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=101 fmt=11 |
| Category | BR/JMP / JUMP |

Description: Relative call (long). Pushes nextPC as return address, then jumps to nextPC + signed immW.

Notes: The primary function call instruction. Callee must STL14 to save the return address.

## CSR Access

### CSRR  (0x4B)

| Property | Value |
| --- | --- |
| Opcode | 0x4B |
| Length | 2 byte(s) |
| Stack Effect | … → …, CSR[csr8] |
| Stack Delta | +1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=010 fmt=11 |
| Category | SYSTEM / CSR |

Description: Read a Control/Status Register. The byte following the opcode is the CSR number (0-255). The CSR value is pushed.

Notes: Serializing (BARRIER=Y). CSRs: 0=MSTATUS, 1=MIE, 2=MIP, 3=MTVEC, 4=MEPC, 5=MCAUSE, 6=MTVAL, 7=MEXT.

### CSRW  (0x4F)

| Property | Value |
| --- | --- |
| Opcode | 0x4F |
| Length | 2 byte(s) |
| Stack Effect | …, x → … |
| Stack Delta | -1 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=011 fmt=11 |
| Category | SYSTEM / CSR |

Description: Write a CSR. The byte following is the CSR number. A is written to the CSR and consumed.

Notes: Serializing. MEXT (CSR 7) is read-only; writes are silently ignored.

## Interrupt Control

### SETI  (0x53)

| Property | Value |
| --- | --- |
| Opcode | 0x53 |
| Length | 2 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=100 fmt=11 |
| Category | SYSTEM / IRQ |

Description: Enable interrupt sources. MIE |= mask8. The byte following is a bitmask of interrupt sources to enable.

Notes: Bit 0=SW, Bit 1=TIMER, Bit 2=EXT.

### CLRI  (0x57)

| Property | Value |
| --- | --- |
| Opcode | 0x57 |
| Length | 2 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | No |
| Encoding | b7=0 f2=10 f3=101 fmt=11 |
| Category | SYSTEM / IRQ |

Description: Disable interrupt sources. MIE &= ~mask8.

## System Control

### EI  (0x5B)

| Property | Value |
| --- | --- |
| Opcode | 0x5B |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=110 fmt=11 |
| Category | SYSTEM / CTRL |

Description: Global interrupt enable. Sets MSTATUS.MIE = 1.

Notes: Barrier instruction. Typically placed at the end of an interrupt handler prologue.

### DI  (0x5F)

| Property | Value |
| --- | --- |
| Opcode | 0x5F |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=10 f3=111 fmt=11 |
| Category | SYSTEM / CTRL |

Description: Global interrupt disable. Clears MSTATUS.MIE = 0.

Notes: Use before critical sections that must not be interrupted.

### IRET  (0x63)

| Property | Value |
| --- | --- |
| Opcode | 0x63 |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=000 fmt=11 |
| Category | SYSTEM / CTRL |

Description: Return from trap/interrupt. PC = MEPC; MSTATUS.MIE = MSTATUS.MPIE; MPIE = 1.

Notes: Must be the last instruction of every trap/interrupt handler.

### HLT  (0xFF)

| Property | Value |
| --- | --- |
| Opcode | 0xFF |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=1 f2=11 f3=111 fmt=11 |
| Category | SYSTEM / CTRL |

Description: Halt until interrupt. CPU enters low-power wait state. Wakes when (MIP & MIE) != 0 and MSTATUS.MIE = 1.

Notes: WFI-like (Wait For Interrupt). Used in idle loops to save power/reduce simulation cycles.

## Traps

### ECALL  (0x67)

| Property | Value |
| --- | --- |
| Opcode | 0x67 |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=001 fmt=11 |
| Category | SYSTEM / TRAP |

Description: Environment call. Triggers synchronous trap with cause TRAP_ECALL (0x08). MEPC points to the ECALL instruction.

Notes: The system call mechanism. ABI: set up arguments in w0-w3, then ECALL. Handler dispatches based on w0.

### EBREAK  (0x6B)

| Property | Value |
| --- | --- |
| Opcode | 0x6B |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=010 fmt=11 |
| Category | SYSTEM / TRAP |

Description: Breakpoint. Triggers synchronous trap with cause TRAP_EBREAK (0x03). MEPC points to the EBREAK instruction.

Notes: Used for software breakpoints. A debugger/monitor replaces an instruction with EBREAK, then inspects state on trap entry.

## Memory Ordering

### FENCE  (0x6F)

| Property | Value |
| --- | --- |
| Opcode | 0x6F |
| Length | 1 byte(s) |
| Stack Effect | … → … |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=011 fmt=11 |
| Category | SYSTEM / ORDER |

Description: Memory ordering barrier. All prior loads/stores complete before any subsequent loads/stores.

Notes: Required for MMIO and DMA ordering. On simple in-order cores, FENCE may be a NOP but remains a serialization point.

## Extension Escape

### ESC  (0x7F)

| Property | Value |
| --- | --- |
| Opcode | 0x7F |
| Length | 2+ byte(s) |
| Stack Effect |  |
| Stack Delta | 0 |
| Barrier | Yes (scheduling boundary) |
| Encoding | b7=0 f2=11 f3=111 fmt=11 |
| Category | PREFIX / ESC |

Description: Extension escape. The next byte is an EXT0 opcode. Within EXT0, byte 0xFF chains to EXT1 (XESC).

Notes: BASE never changes; all future instructions go through ESC. EXT0 currently defines R8BLK (MOVB/MOVW/FILLB/FILLW).

## Workspace Local Load/Store

R8 provides 16 dedicated 1-byte local load instructions (LDL0-LDL15) and 16 local store instructions (STL0-STL15), plus extended variants LDLX and STLX for slots 16-255.

| Opcodes | Mnemonic | Len | Stack Effect | Description |
| --- | --- | --- | --- | --- |
| 0x03..0x3F (step 4) | LDL0-LDL15 | 1 | +1: … → …, wksp[n] | Push workspace slot n onto the operand stack. |
| 0x83..0xBF (step 4) | STL0-STL15 | 1 | -1: …, x → … | Pop TOS and store into workspace slot n. |
| 0x7B | LDLX imm8 | 2 | +1: … → …, wksp[i] | Push workspace slot i (0-255) onto the stack. |
| 0xFB | STLX imm8 | 2 | -1: …, x → … | Pop TOS and store into workspace slot i (0-255). |

The 1-byte variants (LDL0-LDL15, STL0-STL15) use the fmt=11 column with b7 selecting load (b7=0) vs store (b7=1). The register index is encoded in funct3 and funct2 as a 4-bit field: slot = bits[6:2] >> 2 = (opcode >> 2) & 0x0F.

# Part III — Platform Specification

## Chapter 10: Trap and Interrupt Model

### 10.1 Trap Entry (Hardware Sequence)

On any synchronous trap or asynchronous interrupt, hardware atomically performs:

1) mtval  <- trap-specific value (or 0 for interrupts)

2) mepc   <- PC of trapping instruction (sync) or nextPC (async)

3) mcause <- (isInterrupt ? MSB : 0) | causeCode

4) mstatus.MPIE <- mstatus.MIE; mstatus.MIE <- 0

5) PC <- mtvec

No registers are automatically saved. The handler must preserve A/B/C and any workspace registers it uses.

Interrupt safety: ADDC/SUBB carry/borrow values are on the operand stack, not in hidden flags. An interrupt between carry-chain instructions is safe because the handler's save/restore of A/B/C preserves the carry.

### 10.2 Trap Causes

| Code | Name | Condition |
| --- | --- | --- |
| 0x02 | TRAP_ILLEGAL_INSN | Unknown opcode or width-guarded instruction on unsupported WORD_BITS |
| 0x03 | TRAP_EBREAK | EBREAK instruction (software breakpoint) |
| 0x08 | TRAP_ECALL | ECALL instruction (system call) |
| 0x18 | TRAP_DIV_ZERO | DIV/REM/DIVU/REMU with divisor == 0 |

### 10.3 Interrupt Priority

Effective pending = (MIP & MIE) gated by MSTATUS.MIE. Priority: SW (highest) > TIMER > EXT (lowest).

### 10.4 CSR Register Map

| CSR | Name | R/W | Description |
| --- | --- | --- | --- |
| 0x00 | MSTATUS | R/W | MIE (global enable) and MPIE (saved enable) |
| 0x01 | MIE | R/W | Per-source interrupt enable: SW(0), TIMER(1), EXT(2) |
| 0x02 | MIP | R/W* | Pending bits. SW direct-write; TIMER/EXT write-1-to-clear |
| 0x03 | MTVEC | R/W | Trap vector base address (direct mode) |
| 0x04 | MEPC | R/W | Saved PC on trap/interrupt entry |
| 0x05 | MCAUSE | R/W | Cause code; MSB=1 for interrupt |
| 0x06 | MTVAL | R/W | Trap value (optional) |
| 0x07 | MEXT | R/O | Extension discovery bitmask. bit0=R8BLK |

## Chapter 11: EXT0 Block Operations (R8BLK)

R8BLK provides restartable block copy/fill operations using workspace registers as operands. Required for the R8-CONSOLE profile.

| EXT0 Byte | Mnemonic | w0 | w1 | w2 | Unit |
| --- | --- | --- | --- | --- | --- |
| 0x01 | MOVB | dst (updated) | src (updated) | count (decr to 0) | bytes |
| 0x05 | MOVW | dst (updated) | src (updated) | count (decr to 0) | words |
| 0x09 | FILLB | dst (updated) | value (unchanged) | count (decr to 0) | bytes |
| 0x0D | FILLW | dst (updated) | value (unchanged) | count (decr to 0) | words |

Restartable semantics: each instruction performs one element per internal iteration, updates w0/w1/w2, and keeps PC on the instruction until w2==0. Interrupts may occur between iterations. If w2==0 on entry, no memory is accessed.

Direction: forward (low to high). Overlap is undefined (memcpy semantics). Use software backward loop for memmove when dst > src.

## Chapter 12: Memory Ordering

FENCE is a full ordering barrier: all prior loads/stores complete before any subsequent loads/stores. Required for MMIO register writes (e.g., writing VPU parameters then triggering a command) and DMA synchronization.

DMA rule: FENCE before writing START to a device; FENCE after completion/IRQ before consuming DMA-written results.

## Chapter 13: Conformance Profiles

| Profile | Requirements | Notes |
| --- | --- | --- |
| R8-BASE | All 108 BASE opcodes, CSRs 0x00-0x07 | Minimal compliant core. ESC may trap if EXT0 not implemented. |
| R8-CONSOLE | R8-BASE + EXT0:R8BLK (MOVB/MOVW/FILLB/FILLW) | Target for BIOS+BASIC+games. MEXT.bit0 must be 1. |

# Part IV — Encoding Reference

## Chapter 14: Opcode Byte Layout

Every R8 instruction begins with a single 8-bit opcode byte decoded using four stable fields:

bit 7   | bits 6:5 | bits 4:2 | bits 1:0

b7     |   f2     |   f3     |   fmt

| fmt | Class | Typical Length | Contents |
| --- | --- | --- | --- |
| 00 | OP (ALU/stack) | 1 byte | Arithmetic, logic, compare, shift, stack manipulation, Nintendo primitives |
| 01 | LOAD/STORE/MEM | 1 byte | Memory loads, stores, PUSH, POP |
| 10 | IMM/BRANCH | 1-2 bytes | Immediates (B/U), conditional branches, JR/JALR |
| 11 | LOCAL/SYS/IMMW | 1, 2, or 1+W | Workspace (LDL/STL), system (CSR/IRQ/CTRL), word immediates (I/AIIP/J/JAL), ESC |

## Chapter 15: Opcode Map (Numerical Order)

| Opcode | Mnem | Len | Stk | Bar |
| --- | --- | --- | --- | --- |
| 0x00 | NOP | 1 | 0 | N |
| 0x01 | LD | 1 | 0 | N |
| 0x02 | B | 2 | +1 | N |
| 0x03 | LDL0 | 1 | +1 | N |
| 0x04 | DUP | 1 | +1 | N |
| 0x05 | LB | 1 | 0 | N |
| 0x06 | U | 2 | +1 | N |
| 0x07 | LDL1 | 1 | +1 | N |
| 0x08 | SWAP | 1 | 0 | N |
| 0x09 | LU | 1 | 0 | N |
| 0x0B | LDL2 | 1 | +1 | N |
| 0x0C | DROP1 | 1 | -1 | N |
| 0x0D | LH | 1 | 0 | N |
| 0x0F | LDL3 | 1 | +1 | N |
| 0x10 | DROP2 | 1 | -2 | N |
| 0x11 | LHU | 1 | 0 | N |
| 0x13 | LDL4 | 1 | +1 | N |
| 0x14 | I2B | 1 | 0 | N |
| 0x15 | L32 | 1 | 0 | N |
| 0x17 | LDL5 | 1 | +1 | N |
| 0x19 | L32U | 1 | 0 | N |
| 0x1B | LDL6 | 1 | +1 | N |
| 0x1F | LDL7 | 1 | +1 | N |
| 0x20 | ADD | 1 | -1 | N |
| 0x21 | ST | 1 | -2 | N |
| 0x22 | BEQ | 2 | -2 | Y |
| 0x23 | LDL8 | 1 | +1 | N |
| 0x24 | SUB | 1 | -1 | N |
| 0x25 | SB | 1 | -2 | N |
| 0x26 | BNE | 2 | -2 | Y |
| 0x27 | LDL9 | 1 | +1 | N |
| 0x28 | MUL | 1 | -1 | N |
| 0x29 | SH | 1 | -2 | N |
| 0x2A | BLT | 2 | -2 | Y |
| 0x2B | LDL10 | 1 | +1 | N |
| 0x2C | DIV | 1 | -1 | N |
| 0x2D | S32 | 1 | -2 | N |
| 0x2E | BLTU | 2 | -2 | Y |
| 0x2F | LDL11 | 1 | +1 | N |
| 0x30 | REM | 1 | -1 | N |
| 0x32 | BGE | 2 | -2 | Y |
| 0x33 | LDL12 | 1 | +1 | N |
| 0x34 | INC | 1 | 0 | N |
| 0x36 | BGEU | 2 | -2 | Y |
| 0x37 | LDL13 | 1 | +1 | N |
| 0x38 | DEC | 1 | 0 | N |
| 0x3A | BEQZ | 2 | -1 | Y |
| 0x3B | LDL14 | 1 | +1 | N |
| 0x3C | NEG | 1 | 0 | N |
| 0x3E | BNEZ | 2 | -1 | Y |
| 0x3F | LDL15 | 1 | +1 | N |
| 0x40 | AND | 1 | -1 | N |
| 0x41 | POP | 1 | +1 | N |
| 0x42 | JR | 1 | -1 | Y |

| Opcode | Mnem | Len | Stk | Bar |
| --- | --- | --- | --- | --- |
| 0x43 | I | 1+W | +1 | N |
| 0x44 | OR | 1 | -1 | N |
| 0x45 | PUSH | 1 | -1 | N |
| 0x46 | JALR | 1 | 0 | Y |
| 0x47 | AIIP | 1+W | +1 | N |
| 0x48 | XOR | 1 | -1 | N |
| 0x4B | CSRR | 2 | +1 | Y |
| 0x4C | INV | 1 | 0 | N |
| 0x4F | CSRW | 2 | -1 | Y |
| 0x50 | SEQ | 1 | -1 | N |
| 0x53 | SETI | 2 | 0 | N |
| 0x54 | SNE | 1 | -1 | N |
| 0x57 | CLRI | 2 | 0 | N |
| 0x5B | EI | 1 | 0 | Y |
| 0x5F | DI | 1 | 0 | Y |
| 0x60 | SHL | 1 | -1 | N |
| 0x63 | IRET | 1 | 0 | Y |
| 0x64 | SHR | 1 | -1 | N |
| 0x67 | ECALL | 1 | 0 | Y |
| 0x68 | SAR | 1 | -1 | N |
| 0x6B | EBREAK | 1 | 0 | Y |
| 0x6C | SLT | 1 | -1 | N |
| 0x6F | FENCE | 1 | 0 | Y |
| 0x70 | SLTU | 1 | -1 | N |
| 0x73 | J | 1+W | 0 | Y |
| 0x77 | JAL | 1+W | +1 | Y |
| 0x7B | LDLX | 2 | +1 | N |
| 0x7F | ESC | 2+ | 0 | Y |
| 0x80 | ADDC | 1 | 0 | N |
| 0x83 | STL0 | 1 | -1 | N |
| 0x84 | SUBB | 1 | 0 | N |
| 0x87 | STL1 | 1 | -1 | N |
| 0x88 | MULH | 1 | -1 | N |
| 0x8B | STL2 | 1 | -1 | N |
| 0x8C | MULHU | 1 | -1 | N |
| 0x8F | STL3 | 1 | -1 | N |
| 0x90 | CLZ | 1 | 0 | N |
| 0x93 | STL4 | 1 | -1 | N |
| 0x97 | STL5 | 1 | -1 | N |
| 0x9B | STL6 | 1 | -1 | N |
| 0x9F | STL7 | 1 | -1 | N |
| 0xA2 | BRA | 2 | 0 | Y |
| 0xA3 | STL8 | 1 | -1 | N |
| 0xA7 | STL9 | 1 | -1 | N |
| 0xAB | STL10 | 1 | -1 | N |
| 0xAC | DIVU | 1 | -1 | N |
| 0xAF | STL11 | 1 | -1 | N |
| 0xB0 | REMU | 1 | -1 | N |
| 0xB3 | STL12 | 1 | -1 | N |
| 0xB7 | STL13 | 1 | -1 | N |
| 0xBB | STL14 | 1 | -1 | N |
| 0xBF | STL15 | 1 | -1 | N |
| 0xFB | STLX | 2 | -1 | N |
| 0xFF | HLT | 1 | 0 | Y |

# Appendix A: Micro-Fusion Rules

Load->ALU: LD/LB/LU immediately followed by a binary ALU op (ADD, SUB, AND, OR, XOR, SEQ, SNE, SLT, SLTU, SHL, SHR, SAR) may fuse internally.

Imm->ALU: B/U/I immediately followed by a binary ALU op may fuse as an op-immediate micro-op.

Address-generation fusion (recommended): (B|U) off8; ADD; LD*/ST* may fuse into a single AGU+MEM micro-op, recovering struct/array access performance.

Never fuse across BARRIER: branches, jumps, CSR, traps, FENCE, IRET, HLT, ESC.

# Appendix B: Quick Reference Card

108 BASE opcodes. 148 RESERVED. 5 EXT0 (R8BLK + XESC). Boolean: false=0, true=1. NOT(x) = B 1; XOR. Little-endian. CLZ(0) = WORD_BITS. Division truncates toward zero. Shifts mask by WORD_BITS-1.

# Appendix C: Assembler Encoding Notes

The r8as assembler uses JSON architecture spec files (r816.json, r824.json, r832.json) that encode every opcode as bit patterns. Branch offsets use IP-relative fixups. The workspace register w15 is aliased as 'sp'. EXT0 ops (movb/movw/fillb/fillw) are assembled as two-byte sequences with the ESC prefix included automatically.