# R8 ISA + Platform Specification (v13 FINAL)

BASE v9 (Nintendo primitives + CLZ) • R8BLK required capability profile (encoded in EXT0) • ABI v0.1 • Trap/IRQ v0.1 • Memory model v0.1 • Fusion Appendix

Spreadsheet: R8_ISA_BASE_v9_FINAL_profiles.xlsx • Generated: 2026-03-11

# 0. Session goal and design targets

Start from an 8‑bit, fielded opcode that is easy to decode in FPGA (grouped by fmt=bits[1:0], then b7, then funct3=bits[4:2], funct2=bits[6:5]).

Keep the architectural model JVM‑like: word‑sized operand stack (Category‑1 values), with A/B/C as a cache only; no flags, no mode bits.

Nintendo rule: highest ROI primitives in BASE; everything else either micro‑fuses (implementation) or moves to extension pages (ESC).

Modern research target: clean OoO/microbatching experiments (deterministic decode, explicit barriers, restartable long ops).

Asynchronous interrupts must be cheap/deterministic and pipeline-friendly (RISC‑V‑inspired CSRs + direct MTVEC).

Games/platform focus: memcpy/memset/string work must be feasible without big silicon (R8BLK) while keeping room for DMA for large transfers.

## 0.1 Conformance profiles

R8 defines a stable BASE ISA and uses ESC to enter extension pages. Profiles specify which extensions are required.

| Profile | Required instructions | Required CSRs | Notes |
| --- | --- | --- | --- |
| R8-BASE | All BASE opcodes in the spreadsheet | CSR 0x00–0x07 (MEXT may be 0) | Minimal compliant core; may trap ESC into EXT0 if EXT0 not implemented. |
| R8-CONSOLE | R8-BASE + EXT0:R8BLK (MOVB/MOVW/FILLB/FILLW) | CSR 0x00–0x07 with MEXT.bit0=1 | Target profile for BIOS+BASIC+games; BLK is mandatory capability but encoded in EXT0 to preserve BASE opcode map. |

# 1. Architectural model

R8 is a functional operand-stack ISA. Architectural state is the operand stack (native WORD values), workspace locals (wksp[0..15] + extended via LDLX/STLX), and CSRs.

A/B/C are an implementation cache of the top of the operand stack; instruction semantics are specified as stack transforms.

RISC-inspired: simple fielded decode, minimal addressing modes, base+extensions discipline.

Nintendo rule: only high-ROI primitives in BASE (ADDC/SUBB, MULH/MULHU, CLZ). No architectural flags.

# 2. Encoding and extension mechanism

Opcode byte fields: fmt=bits[1:0], funct3=bits[4:2], funct2=bits[6:5], b7=bit[7].

Spreadsheet layout mirrors decode: primary grouping by fmt, then b7, then funct3, then funct2 (original ‘bit grid’ style).

Instruction length is known from fmt and subfields; no multi-byte prefix scanning is required.

ESC (0x7F) enters EXT0. Within an extension page, 0xFF is reserved for chaining (XESC) to deeper pages.

Micro-fusion is permitted but never required; it never crosses BARRIER instructions.

# 3. BASE instruction set (by category)

## OP — Stack & ALU

### ARITH

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x20 | ADD | 1 | -1 | N | Stack: …, x, y → …, x+y |  |
| 0x24 | SUB | 1 | -1 | N | Stack: …, x, y → …, x-y |  |
| 0x28 | MUL | 1 | -1 | N | Stack: …, x, y → …, x*y |  |
| 0x2C | DIV | 1 | -1 | N | Stack: …, x, y → …, x/y | Trap if y==0. REM: remainder sign follows dividend (like Java/C). REM by zero traps TRAP_DIV_ZERO. |
| 0x30 | REM | 1 | -1 | N | Stack: …, x, y → …, x%y | Trap if y==0. |
| 0x34 | INC | 1 | 0 | N | Stack: …, x → …, x+1 |  |
| 0x38 | DEC | 1 | 0 | N | Stack: …, x → …, x-1 |  |
| 0x3C | NEG | 1 | 0 | N | Stack: …, x → …, -x |  |
| 0x80 | ADDC | 1 | 0 | N | Stack: …, x, y → …, carry, sum | Carry-on-stack primitive (no flags). carry is a normal word value (0/1) below sum so STL can store sum and leave carry for chaining. |
| 0x84 | SUBB | 1 | 0 | N | Stack: …, x, y → …, borrow, diff | Borrow-on-stack primitive (no flags). borrow is 0/1 below diff. |
| 0x88 | MULH | 1 | -1 | N | Stack: …, x, y → …, hi | Signed upper-half multiply. Use with MUL (low half) to form 2×WORD products in software. |
| 0x8C | MULHU | 1 | -1 | N | Stack: …, x, y → …, hi | Unsigned upper-half multiply. Use with MUL (low half) for 16×16→32 (R816) and generally 2×WORD products. |
| 0xAC | DIVU | 1 | -1 | N | Stack: …, x, y → …, u(x)/u(y) | Unsigned divide. Trap if y==0. DIVU: unsigned division. DIVU by zero traps TRAP_DIV_ZERO. |
| 0xB0 | REMU | 1 | -1 | N | Stack: …, x, y → …, u(x)%u(y) | Unsigned remainder. Trap if y==0. REMU: unsigned remainder. REMU by zero traps TRAP_DIV_ZERO. |

### BIT

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x90 | CLZ | 1 | 0 | N | Stack: …, x → …, n | Count leading zeros (priority encode). High ROI for soft-float normalization, log2, allocators, bitmaps. CLZ(0)=WORD_BITS (normative). |

### CMP

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x50 | SEQ | 1 | -1 | N | Stack: …, x, y → …, (x==y)?1:0 |  |
| 0x54 | SNE | 1 | -1 | N | Stack: …, x, y → …, (x!=y)?1:0 |  |
| 0x6C | SLT | 1 | -1 | N | Stack: …, x, y → …, (x<y)?1:0 |  |
| 0x70 | SLTU | 1 | -1 | N | Stack: …, x, y → …, (u(x)<u(y))?1:0 |  |

### LOGIC

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x40 | AND | 1 | -1 | N | Stack: …, x, y → …, x&y |  |
| 0x44 | OR | 1 | -1 | N | Stack: …, x, y → …, x¦y |  |
| 0x48 | XOR | 1 | -1 | N | Stack: …, x, y → …, x^y |  |
| 0x4C | INV | 1 | 0 | N | Stack: …, x → …, ~x |  |

### SHIFT

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x60 | SHL | 1 | -1 | N | Stack: …, x, s → …, (x<<s) | mask = (WORD_BITS-1). |
| 0x64 | SHR | 1 | -1 | N | Stack: …, x, s → …, (u(x)>>>s) | mask = (WORD_BITS-1). |
| 0x68 | SAR | 1 | -1 | N | Stack: …, x, s → …, (x>>s) | mask = (WORD_BITS-1). |

### STACK

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00 | NOP | 1 | 0 | N | Stack: … → … |  |
| 0x04 | DUP | 1 | 1 | N | Stack: …, x → …, x, x |  |
| 0x08 | SWAP | 1 | 0 | N | Stack: …, x, y → …, y, x |  |
| 0x0C | DROP1 | 1 | -1 | N | Stack: …, x → … |  |
| 0x10 | DROP2 | 1 | -2 | N | Stack: …, x, y → … |  |
| 0x14 | I2B | 1 | 0 | N | Stack: …, x → …, (byte)x |  |

## IMM — Immediates

### IMM8

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x02 | B | 2 | 1 | N | Stack: … → …, imm | Signed 8-bit immediate. |
| 0x06 | U | 2 | 1 | N | Stack: … → …, imm | Unsigned 8-bit immediate. |

### IMMW

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x43 | I | 1+W | 1 | N | Stack: … → …, imm | Signed native-word immediate. |

### PCREL

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x47 | AIIP | 1+W | 1 | N | Stack: … → …, addr | PC-relative address (native word immediate). |

## LOAD — Memory loads

### NARROW

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x05 | LB | 1 | 0 | N | Stack: …, addr → …, val |  |
| 0x09 | LU | 1 | 0 | N | Stack: …, addr → …, val |  |
| 0x0D | LH | 1 | 0 | N | Stack: …, addr → …, val | Valid when WORD_BYTES>=4 and even. |
| 0x11 | LHU | 1 | 0 | N | Stack: …, addr → …, val | Valid when WORD_BYTES>=4 and even. |
| 0x15 | L32 | 1 | 0 | N | Stack: …, addr → …, val | Valid when WORD_BYTES>4 (e.g., 64-bit). |
| 0x19 | L32U | 1 | 0 | N | Stack: …, addr → …, val | Valid when WORD_BYTES>4 (e.g., 64-bit). |

### WORD

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x01 | LD | 1 | 0 | N | Stack: …, addr → …, val |  |

## STORE — Memory stores

### NARROW

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x25 | SB | 1 | -2 | N | Stack: …, addr, val → … |  |
| 0x29 | SH | 1 | -2 | N | Stack: …, addr, val → … | Valid when WORD_BYTES>=4 and even. |
| 0x2D | S32 | 1 | -2 | N | Stack: …, addr, val → … | Valid when WORD_BYTES>4 (e.g., 64-bit). |

### WORD

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x21 | ST | 1 | -2 | N | Stack: …, addr, val → … |  |

## MEM — Stack ↔ memory (SP)

### STACKMEM

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x41 | POP | 1 | 1 | N | Stack: … → …, val | Reads native word size. |
| 0x45 | PUSH | 1 | -1 | N | Stack: …, x → … | Writes native word size. |

## BR/JMP — Control flow

### BRANCH

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x22 | BEQ | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x26 | BNE | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x2A | BLT | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x2E | BLTU | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x32 | BGE | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x36 | BGEU | 2 | -2 | Y | Stack: …, x, y → … | off = sext8(imm8). |
| 0x3A | BEQZ | 2 | -1 | Y | Stack: …, x → … | off = sext8(imm8). |
| 0x3E | BNEZ | 2 | -1 | Y | Stack: …, x → … | off = sext8(imm8). |
| 0xA2 | BRA | 2 | 0 | Y | Stack: … → … | Unconditional relative branch (imm8). Uses b7=1 variant. |

### JUMP/CALL

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x42 | JR | 1 | -1 | Y | Stack: …, addr → … | Jump register (consumes addr). |
| 0x46 | JALR | 1 | 0 | Y | Stack: …, addr → …, ret | Call register. Replaces addr with return address on stack (STACKΔ=0). |
| 0x73 | J | 1+W | 0 | Y | Stack: … → … | Relative jump (native word immediate). |
| 0x77 | JAL | 1+W | 1 | Y | Stack: … → …, ret | Relative call (push return address). |

## LOCAL — Workspace locals

### WKSP

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x03 | LDL0 | 1 | 1 | N | Stack: … → …, wksp[0] | Local load (1-byte). |
| 0x07 | LDL1 | 1 | 1 | N | Stack: … → …, wksp[1] | Local load (1-byte). |
| 0x0B | LDL2 | 1 | 1 | N | Stack: … → …, wksp[2] | Local load (1-byte). |
| 0x0F | LDL3 | 1 | 1 | N | Stack: … → …, wksp[3] | Local load (1-byte). |
| 0x13 | LDL4 | 1 | 1 | N | Stack: … → …, wksp[4] | Local load (1-byte). |
| 0x17 | LDL5 | 1 | 1 | N | Stack: … → …, wksp[5] | Local load (1-byte). |
| 0x1B | LDL6 | 1 | 1 | N | Stack: … → …, wksp[6] | Local load (1-byte). |
| 0x1F | LDL7 | 1 | 1 | N | Stack: … → …, wksp[7] | Local load (1-byte). |
| 0x23 | LDL8 | 1 | 1 | N | Stack: … → …, wksp[8] | Local load (1-byte). |
| 0x27 | LDL9 | 1 | 1 | N | Stack: … → …, wksp[9] | Local load (1-byte). |
| 0x2B | LDL10 | 1 | 1 | N | Stack: … → …, wksp[10] | Local load (1-byte). |
| 0x2F | LDL11 | 1 | 1 | N | Stack: … → …, wksp[11] | Local load (1-byte). |
| 0x33 | LDL12 | 1 | 1 | N | Stack: … → …, wksp[12] | Local load (1-byte). |
| 0x37 | LDL13 | 1 | 1 | N | Stack: … → …, wksp[13] | Local load (1-byte). |
| 0x3B | LDL14 | 1 | 1 | N | Stack: … → …, wksp[14] | Local load (1-byte). |
| 0x3F | LDL15 | 1 | 1 | N | Stack: … → …, wksp[15] | Local load (1-byte). |
| 0x7B | LDLX | 2 | 1 | N | Stack: … → …, wksp[i] | Extended local load by imm8 index. Keeps LDL0..15 1-byte hot path; expands locals to 0..255. |
| 0x83 | STL0 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x87 | STL1 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x8B | STL2 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x8F | STL3 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x93 | STL4 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x97 | STL5 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x9B | STL6 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0x9F | STL7 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xA3 | STL8 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xA7 | STL9 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xAB | STL10 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xAF | STL11 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xB3 | STL12 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xB7 | STL13 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xBB | STL14 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xBF | STL15 | 1 | -1 | N | Stack: …, x → … | Local store (1-byte). |
| 0xFB | STLX | 2 | -1 | N | Stack: …, x → … | Extended local store by imm8 index. Use for wksp[16..255]. |

## SYSTEM — CSRs, traps, ordering

### CSR

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x4B | CSRR | 2 | 1 | Y | Stack: … → …, val | Read CSR (csr8 follows). Serializing/barrier. |
| 0x4F | CSRW | 2 | -1 | Y | Stack: …, x → … | Write CSR (csr8 follows). Serializing/barrier. |

### CTRL

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x5B | EI | 1 | 0 | Y | Stack: … → … | Global interrupt enable. |
| 0x5F | DI | 1 | 0 | Y | Stack: … → … | Global interrupt disable. |
| 0x63 | IRET | 1 | 0 | Y | Stack: … → … | Return from trap/interrupt. |
| 0xFF | HLT | 1 | 0 | Y | Stack: … → … | WFI-like. Wakes when (MIP & MIE)!=0 and MSTATUS.MIE=1. |

### IRQ

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x53 | SETI | 2 | 0 | N | Stack: … → … | Enable interrupt sources (mask8 follows). |
| 0x57 | CLRI | 2 | 0 | N | Stack: … → … | Disable interrupt sources (mask8 follows). |

### ORDER

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x6F | FENCE | 1 | 0 | Y | Stack: … → … | Memory-mapped I/O ordering barrier. Orders all prior loads/stores before any following loads/stores (DMA/MMIO-safe). |

### TRAP

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x67 | ECALL | 1 | 0 | Y | Stack: … → … | Environment call. |
| 0x6B | EBREAK | 1 | 0 | Y | Stack: … → … | Breakpoint. |

## PREFIX — Extension control

### ESC

| Opcode | Mnemonic | Len | StackΔ | Barrier | Stack effect | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0x7F | ESC | 2+ | 0 | Y |  | ESC enters EXT0. In EXT pages, 0xFF chains to next level. |

# 4. Normative corner cases and invariants

Operand stack underflow (insufficient operands for an instruction) traps as ILLEGAL_INSN (TRAP_ILLEGAL_INSN) in BASE v8. Future dedicated causes are reserved.

Division: DIV truncates toward zero; REM follows the dividend sign (Java/C rule). DIV/REM by zero trap TRAP_DIV_ZERO.

Unsigned division/remainder: DIVU/REMU operate on unsigned WORDs; divide-by-zero traps TRAP_DIV_ZERO.

Shifts: SHL/SHR/SAR mask shift count: k = A & (WORD_BITS-1).

CLZ(0) is defined as WORD_BITS (normative).

Memory endianness is little-endian for all multi-byte loads/stores.

Alignment: unaligned accesses are permitted; implementations may execute them as byte-wise accesses. (Software must not assume traps.)

# 5. ABI v0.1 (calling convention)

## 5.1 Workspace roles (normative)

| wksp | Role |
| --- | --- |
| w15 | SP — memory stack pointer for PUSH/POP. |
| w14 | LR — link register (return PC). |
| w0..w3 | arg0..arg3 / scratch / return in w0. |
| w4..w13 | scratch/locals. Caller-saved in ABI v0.1; recommended as handler scratch. |

## 5.2 Call/return

On entry, JAL/JALR places return PC on operand stack (A). Callee stores it into LR:

STL14    ; LR = return PC

Return:

LDL14 ; JR

## 5.3 Trap/IRQ preservation contract

Trap/IRQ entry does not push any registers automatically.

Handlers must preserve architectural state they do not intend to consume: operand stack contents (A/B/C) and any workspace registers that must remain live across the interrupt.

ABI v0.1 requires handlers to preserve w0..w3, w14, w15. Handlers may freely use w4..w13.

# 6. Traps/IRQs v0.1 (normative)

## 6.1 Trap entry

On any trap or interrupt, hardware performs:
1) mtval ← tval (or 0 for interrupts)
2) mepc  ← epc   (trap: PC of trapping instruction; IRQ: next PC)
3) mcause← (isIRQ?MSB:0) | cause
4) mstatus.MPIE ← mstatus.MIE; mstatus.MIE ← 0
5) PC ← mtvec

## 6.2 IRQ priority

Priority order: SW → TIMER → EXT. Effective pending = (MIP & MIE) gated by MSTATUS.MIE.

## 6.3 Extension discovery CSR

CSR 0x07 MEXT is read-only and reports implemented extensions:

bit0=R8BLK (must be 1 for R8-CONSOLE); bit1=FP16; bit2=FP32; bit3=ATOM; bit4=SIMD (reserved).

# 7. Memory ordering v0.1

FENCE is a full ordering barrier for DMA/MMIO-visible ordering: all prior loads/stores complete before any subsequent loads/stores.

In in-order single-core systems without DMA, FENCE may act as a NOP but remains a serialization point for compatibility.

DMA rule: FENCE before writing START; FENCE after completion/IRQ before consuming results.

# 8. R8BLK v0.1 (required for R8-CONSOLE; encoded in EXT0)

Borrowed from Z80 LDIR/LDDR (restartable repeat) and AVR pointer registers (fixed pointer/count regs), adapted to R8 workspace.

## 8.1 Operands table (normative)

| Instr | w0 | w1 | w2 | Unit |
| --- | --- | --- | --- | --- |
| MOVB | dst (updated) | src (updated) | count→0 | bytes |
| MOVW | dst (updated) | src (updated) | count→0 | words |
| FILLB | dst (updated) | value (unchanged) | count→0 | bytes |
| FILLW | dst (updated) | value (unchanged) | count→0 | words |

## 8.2 Restartable semantics (normative)

Entry: if w2==0, instruction completes immediately (no memory access).

Otherwise, the instruction performs one element per iteration, updates w0/w1/w2 accordingly, and repeats itself (PC remains on the instruction) until w2==0, then falls through.

Interrupts may occur between iterations. Implementations must check for pending interrupts between iterations often enough to avoid unbounded latency.

These instructions are BARRIER=Y.

## 8.3 Direction/overlap/alignment

MOVB/MOVW copy forward (low→high). Overlap is undefined (memcpy semantics).

MOVW/FILLW step size is WORD_BYTES per iteration (w0/w1 advance by WORD_BYTES). Implementations may internally byte-copy for misalignment without changing architectural increments.

MOVW count is in words; callers converting byte counts must divide by WORD_BYTES.

## 8.4 Stack-friendly macro (recommended)

; memcpy(dst, src, nbytes) using MOVB (EXT0 byte 0x00)
STL2        ; n -> w2
STL1        ; src -> w1
STL0        ; dst -> w0
ESC
.byte 0x00  ; MOVB

# 9. Fusion appendix (implementation-only, non-normative)

These fusions may be implemented as single macro-ops. They must not change architectural results and must not cross BARRIER instructions.

Load→ALU: LD/LB/LU/LH/LHU/L32/L32U immediately followed by a binary ALU op (ADD/SUB/AND/OR/XOR/SEQ/SNE/SLT/SLTU) may fuse.

Imm→ALU: B/U/I immediately followed by a binary ALU op may fuse.

Offset-load fusion (recommended): (B|U) off8 ; ADD ; LD* may fuse into an address-generation + load macro-op (equivalent to LD with base+off). Same for stores: (B|U) off8 ; ADD ; ST*.

# 10. Games, bulk memory, and DMA guidance (non-normative)

R8 targets ‘console practical’ efficiency on small FPGA cores.

R8BLK covers short/medium copies and string/memory primitives (where DMA setup overhead dominates).

DMA is still valuable for large transfers (kilobytes+), streaming VRAM, audio buffers, and decompressor output.

If DMA exists, require FENCE before ‘START’ writes and after completion/IRQ before consuming results.

# 11. Floating-point on R816 (soft-float first) (non-normative)

BASE does not mandate hardware FP. Soft-float libraries implement FP16/FP32 using integer ops; CLZ is included in BASE to make normalization practical.

Normative corner case: CLZ(0)=WORD_BITS.

FP32 on R816 uses two 16-bit words for mantissa; normalization uses CLZ on hi word (or 16+CLZ(lo) when hi==0) plus software multiword shift.

If profiling demands hardware FP later: define EXT0_FP16/EXT0_FP32 and expose via MEXT bits without changing BASE.

# 12. OoO / microbatching notes (implementation guidance, non-normative)

This ISA was shaped to support modern research cores:

Fielded first-byte decode → simple instruction queues and micro-batch decoders.

BARRIER ops (branches/jumps, traps, FENCE, BLK) → clean scheduling/fusion boundaries.

Prefer micro-fusion (LD→ALU, IMM→ALU, and common address+load sequences) over new opcodes.

R8BLK is restartable by construction (state in workspace). Treat as long-latency µop interruptible between element iterations.

# 13. Extension roadmap sketch (non-normative)

Keep BASE frozen; grow via EXT pages and/or micro-fusion.

EXT0_MEM: optional load-with-offset forms if real compiler output shows repeated patterns that micro-fusion cannot cover.

EXT0_BIT: CTZ/POPCNT/bitfield helpers only if profiling justifies them (CLZ is already BASE).

EXT0_ATOM: atomics (CAS/XCHG) if multi-core or DMA coherency requires it.

EXT0_FP16/FP32: optional hardware FP ops (if ever needed).

# Appendix — Toolchain lowerings, debug contract, and measured patterns

## A. Toolchain lowerings (normative for R8‑CONSOLE)

Field base+off8 load/store

Lowering: U/B off8; ADD; LD*   and   U/B off8; ADD; ST*

Console fusion: SHOULD fuse (imm8+ADD) → ADDI; MAY fuse (ADDI+LD/ST) → AGU+MEM μop.

Purpose: keep BASE compact while recovering struct/array performance on console-grade cores.

Long branch lowering

Lowering: invert condition → short skip; then J (word‑imm) or trampoline.

Toolchain: assembler/linker relaxation REQUIRED for production builds.

Purpose: avoid long-branch opcode bloat.

R8BLK call site macro

Macro: … dst src n → STL2; STL1; STL0; ESC; <MOVB/MOVW/FILLB/FILLW>

Rule: BLK ops are BARRIER=Y and restartable; progress state is w0..w2.

Purpose: keep BLK restartability and allow stack-friendly call sites.

## B. Minimal debug contract (normative)

EBREAK

Synchronous trap. MEPC=addr(EBREAK), MCAUSE=TRAP_EBREAK, MTVAL=0; enter MTVEC per standard trap entry.

Used for software breakpoints; debugger/monitor inspects state.

Instruction atomicity

No instruction is interruptible mid-instruction except BLK ops which allow interrupts between elements.

Ensures predictable stepping/interrupt behavior.

State preservation

Trap/IRQ handlers must preserve architectural operand stack (A/B/C) and ABI-required workspace slots.

Always required; BLK does not introduce special save rules.

## C. Measured codegen patterns (R816)

Architectural sequences (fusion may reduce μops but not byte count):

Load field *(p+off8)

R8: U off8; ADD; LD

Cost: 4 bytes / 3 inst (≈ 1 μop with AGU fusion).

Store field *(p+off8)=v

R8: … v p; U off8; ADD; ST

Cost: ≥5 bytes / ≥4 inst (depends on shuffles).